05 November 2003

#### The 2004 Saab 9-3 Sport Saloon

##### Technical Specifications and Performance

## Engine

Petrol

1998 cc. 4-cyl. in-line. Aluminium cylinder head and block. Turbocharged, intercooled. DOHC, 16-valve. Balancer shafts. Dual-mass flywheel.

Diesel

2171c , 4-cyl, in-line. Aluminium cylinder head and cast iron block. Turbocharged, intercooled. SOHC, 16-valve. Balancer shafts. Dual-mass flywheel

Installation

Front, transversely-mounted, hydraulically-damped. Front wheel drive.

## Transmission

Type

5/6 speed manual gearbox. 5 + 2 speed automatic transmission include. Saab Sentronic manual

selection. Steering wheel buttons for gear

selection (automatic option).

Clutch:

Single, dry-plate, diaphragm spring.

###### Steering

Rack and pinion, hydraulic power assistance. Turning circle: 10,8 m (kerb to kerb)

Lock to lock: 2,97 turns

###### Suspension

Front: MacPherson struts and gas shock absorbers. Anti-roll bar. Mounted on hydroformed sub-frame.

Rear: Independent, four link, including toe-link. Coil springs. Gas shock absorbers. Anti-roll bar. Mounted on sub-frame.

Brakes

Hydraulic, dual circuit, vacuum booster. ABS, Mechanical Brake Assist (MBA), Electronic Brake force Distribution (EBD).

Discs:

285 mm ventilated / 278 mm solid steel

302 mm ventilated / 292 mm ventilated steel (210 bhp)

### Wheels/Tyres

Linear:	Alloy, 6.5” x 15”. 195/65R16

Arc:	Alloy, 6.5” x 16”. 215/55R16

Vector:	Alloy, 7.0” x 17”. 225/45R17

Aero:	Alloy, 7.0” x 17”. 225/45R17

### Weights

Kerb:	1440-1590 kg

Load weight: 	390 kg

Max. trailer load:	1600 kg

Distribution:	55% F / 45% R

### Capacities*

Fuel tank:		Petrol 62 litres

Diesel 61 litres

Luggage:	Volume 425 litres

Rear seat splits 60/40%

# Engine and performance data; the 2004 Saab 9-3 Sport Saloon

|  | 2.2 TiD | 1.8t | 2.0t | 2.0T |
| --- | --- | --- | --- | --- |
| Max. power: | 92 KW (125 bhp) @ 4,000 rpm | 110 KW (150 bhp) @ 5,500 rpm | 129 KW (175 bhp) @ 5,500 rpm | 154KW (210 bhp) @ 5,300 rpm |
| Max. torque: | 280 Nm @ 1,500 rpm | 240Nm @ 2,500 rpm | 265Nm @ 2,500 rpm | 300Nm @ 2,500 rpm |
| Bore/stroke: | 84 mm x 98 mm | 86 mm x 86 mm | 86 mm x 86 mm | 86 mm x 86 mm |
| Compression Ratio: | 18.5:1 | 9.5:1 | 9.5:1 | 9.5:1 |
| ### Max. boost pressure: | 1.00 bar | 0.70 bar | 0.70 bar | 0.85 bar |
| Ignition/ Fuel injection: | Bosch. Multi-point Fuel injection. | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. |

| Performance (s) | ## 2.2 TiD | ## 1.8t | ## 2.0t | ## 2.0T |
| --- | --- | --- | --- | --- |
| 0-60 mph | ## M5 10.3 M6 10.3 | ## M5 9.0 ## A5 10.2 | ## M5 8.0 ## A5 9.2 | ## M5 7.4, M6 7.5 A5 8.3 |
| 40-60 mph 4th gear | M5 6.5 M6 6.5 | M5 8.0 A5 - | M5 6.9 A5 - | M5 6.4, M6 6.2 A5 - |
| 50-70 mph 5th gear | ## M5 9.7 M6 9.2 | ## M5 11.5 ## A5 - | ## M5 10.0 ## A5 - | ## M5 9.5, M6 8.5 ## A5 - |
| Top speed mph | M5 124 M6 124 | M5 131 A5 131 | M5 140 A5 137 | M5 146, M6 146 A5 143 |

| ## Fuel consumption, Co2 and Insurance Grouping | ## 2.2 TiD | ## 1.8t | ## 2.0t | ## 2.0T |
| --- | --- | --- | --- | --- |
| City mpg | M5 31.4 M6 31.7 | M5 24.6 A5 21.7 | M5 24.6 A5 21.4 | M5 23.3, M6 23.7 A5 20.2 |
| Highway mpg | ## M5 54.3 M6 54.3 | ## M5 45.6 ## A5 40.4 | ## M5 44.8 ## A5 39.8 | ## M5 44.1, M6 41.5 A5 39.8 |
| Combined mpg | M5 42.8 M6 42.8 | M5 34.9 A5 30.7 | M5 34.0 A5 30.4 | M5 33.2, M6 32.8 A5 29.4 |
| Co2 g/km | ## M5 177 M6 174 | ## M5 195 ## A5 221 | ## M5 199 ## A5 223 | ## M5 202, M6 207 ## A5 232 |
| Insurance group | ## 11E | ## 12E | ## 14E | ## 16E |

M5 = five-speed manual gearbox

M6 = six-speed manual gearbox (2.0T with F40-6 RC gearbox and 3,55 Final Drive)

A5 = five-speed + two Sentronic automatic gearbox