#### The new 2004 Saab 9-3 Convertible

##### Technical Specifications and Performance

## Engine

Layout

1998 cc. 4-cyl. in-line. Aluminium cylinder head and block. Turbocharged, intercooled. DOHC, 16-valve. Balancer shafts. Dual-mass flywheel.

Installation

Front, transversely-mounted, hydraulically-damped. Front wheel drive.

## Transmission

Type

5/6 speed manual gearbox. 5 + 2 speed automatic transmission include. Saab Sentronic manual

selection. Steering wheel buttons for gear

selection (automatic option).

Clutch:

Single, dry-plate, diaphragm spring.

###### Steering

Rack and pinion, hydraulic power assistance. Turning circle: 10,8 m (kerb to kerb)

Lock to lock: 2,97 turns

###### Suspension

Front: MacPherson struts and gas shock absorbers. Anti-roll bar. Mounted on hydroformed sub-frame.

Rear: Independent, four link, including toe-link. Coil springs. Gas shock absorbers. Anti-roll bar. Mounted on sub-frame.

Brakes

Hydraulic, dual circuit, vacuum booster. ABS, Mechanical Brake Assist (MBA), Electronic Brake force Distribution (EBD).

Discs:

285 mm ventilated / 278 mm solid steel (150 & 175 bhp)

302 mm ventilated / 292 mm ventilated steel (210 bhp)

### Wheels/Tyres

Linear:	Alloy, 6.5” x 16”. 215/55R16

Vector:	Alloy, 7.0” x 17”. 215/50R17

Aero:	Alloy, 7.0” x 17”. 215/50R17

### Weights

Kerb:	1580-1685 kg

Load weight: 	390 kg

Max. trailer load:	1600 kg

Distribution:	55% F / 45% R

### Capacities*

Fuel tank:		62 litres

Trunk:	Soft-top up: 352 litres

Soft-top down: 253 litres

Washer Reservoir:    5,0 litres

# Engine and performance data; the new 2004 Saab 9-3 Convertible

|  | 1.8t | 2.0t | 2.0T |
| --- | --- | --- | --- |
| Max. power: | 110 KW (150 bhp) @ 5,500 rpm | 129 KW (175 bhp) @ 5,500 rpm | 154KW (210 bhp) @ 5,300 rpm |
| Max. torque: | 240Nm @ 2,500 rpm | 265Nm @ 2,500 rpm | 300Nm @ 2,500 rpm |
| Bore/stroke: | 86 mm x 86 mm | 86 mm x 86 mm | 86 mm x 86 mm |
| Compression Ratio: | 9.5:1 | 9.5:1 | 9.5:1 |
| ### Max. boost pressure: | 0.70 bar | 0.70 bar | 0.85 bar |
| Ignition/ Fuel injection: | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. | Saab Trionic 8 engine management. Direct ignition. Multi-point fuel injection. |

| Performance (s) | ## 1.8t | ## 2.0t | ## 2.0T |
| --- | --- | --- | --- |
| 0-60 mph | ## M5 10.0 ## A5 11.0 | ## M5 8.5 ## A5 10.0 | ## M6 7.7 A5 9.0 |
| 40-60 mph 4th gear | M5 8.8 A5 - | M5 7.5 A5 - | M6 6.8 A5 - |
| 50-70 mph 5th gear | ## M5 13.4 ## A5 - | ## M5 11.5 ## A5 - | ## M6 9.3 ## A5 - |
| Top speed mph | M5 127 A5 124 | M5 137 A5 134 | M6 143 A5 140 |

| ## Fuel consumption, Co2 and Insurance Grouping | ## 1.8t | ## 2.0t | ## 2.0T |
| --- | --- | --- | --- |
| City mpg | M5 24.1 A5 21.1 | M5 23.7 A5 21.6 | M6 23.2 A5 20.2 |
| Highway mpg | ## M5 42.2 ## A5 37.2 | ## M5 39.8 ## A5 36.7 | ## M6 40.4 A5 37.2 |
| Combined mpg | M5 32.8 A5 29.1 | M5 31.7 A5 29.1 | M6 31.0 A5 28.2 |
| Co2 g/km | ## M5 206 ## A5 233 | ## M5 212 ## A5 232 | ## M6 218 ## A5 235 |
| Insurance group | ## 13E | ## 15E | ## 17E |

M5 = five-speed manual gearbox

M6 = six-speed manual gearbox (2.0T with F40-6 RC gearbox and 3,55 Final Drive)

A5 = five-speed + two Sentronic automatic gearbox