### Model Year 2004, Saab 9-5

### Technical Specifications and Performance

#### Petrol engines

| Engines | 2.0t | 2.3t | 2.3T | 2.3 HOT (Aero) |
| --- | --- | --- | --- | --- |
| #### Displacement | 1,985 cc | 2,290 cc | 2,290 cc | 2,290 cc |
| Cylinders | 4 | 4 | 4 | 4 |
| Bore/stroke | 90/78 mm | 90/90 mm | 90/90 mm | 90/90 mm |
| EEC rating | 150 hp (110 kW) at 5500 rpm. | 185 hp (136kW) at 5500 rpm. | 220 hp (162 kW) at 5500 rpm. | 250 hp (184 kW) at 5300 rpm. |
| EEC peak torque | 240 Nm at 1800-3500 rpm. | 280 Nm at 1800-3500 rpm. | 310 Nm at 1900 - 4500 rpm | 350 Nm at 1900-4000 rpm. Momentarily 370 Nm with the overboost function activated. |
| Compression ratio | 9.3:1 | 9.3:1 | 9.3:1 | 9.3:1 |
| Recommended octane rating | 95 RON | 95 RON | 95 RON | 98 RON |

| Performance | 2.0t | 2.3t | 2.3T | 2.3 HOT (Aero) |
| --- | --- | --- | --- | --- |
| 0-60 mph (sec) Saloon (Estate) | M 9.0 (9.6) A 11.0 (11.7) | M 7.9 (8.4) A 9.0 (9.4) | M 7.5 (8.0) A 8.4 (8.7) | M 6.5 (6.9) A 7.8 (8.1) |
| 40–60 mph in 4th gear (sec) Saloon (Estate) | M 8.4 (8.8) | M 6.4 (7.0) | M 5.9 (6.4) | M 5.0 (5.5) |
| 50-70 mph in 5th gear (sec) Saloon (Estate) | M 12.7 (13.2) | M 10.3 (11.3) | M 9.0 (9.9) | M 7.3 (8.0) |
| #### Top speed mph Saloon (Estate) | M 134 (134) A 131 (127) | M 143 (140) A 140 (137) | M 146 (143) A 143 (140) | M 155 (152) A 155 (152) |

| Fuel consumption mpg | 2.0t |  | 2.3t |  | 2.3T |  | 2.3 HOT(Aero) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | M | A | M | A | M | A | M | A |
| City Saloon | 23.3 | 19.0 | 22.1 | 19.1 | 21.7 | 18.2 | 22.1 | 19.1 |
| Estate | 22.6 | 18.7 | 21.7 | 18.7 | 20.6 | 18.2 | 21.9 | 18.5 |
| Highway Saloon | 42.8 | 37.2 | 42.8 | 39.2 | 43.5 | 38.7 | 44.1 | 41.5 |
| Estate | 40.9 | 35.7 | 41.5 | 37.2 | 39.2 | 38.7 | 42.2 | 39.8 |
| Combined Saloon | 32.8 | 27.4 | 31.7 | 28.20 | 31.7 | 27.4 | 32.1 | 28.8 |
| Estate | 31.4 | 26.6 | 31.0 | 27.4 | 29.4 | 27.4 | 31.4 | 28.0 |

| CO2 Emissions (g/km) | 2.0t |  | 2.3t |  | 2.3T |  | 2.3 HOT (Aero) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | M | A | M | A | M | A | M | A |
| Saloon | 204 | 244 | 212 | 238 | 212 | 247 | 209 | 232 |
| Estate | 214 | 251 | 217 | 246 | 229 | 246 | 214 | 240 |

| Insurance Grouping | 2.0t |  | 2.3t |  | 2.3T |  | 2.3 HOT (Aero) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ###### 13E | 13E | 15E | 15E | 16E | 16E | 17A | 17A |

### Model Year 2004, Saab 9-5

### Technical Specifications and Performance

Diesel engines

| Engines | 3.0 V6 TiD | 2.2 TiD |
| --- | --- | --- |
| Displacement | 2.962 cc | 2.171 cc |
| Cylinders | 6 | 4 |
| Bore/stroke | 87.5/82 mm | 84/98 mm |
| EEC rating | 176 hp (129 kW) at 4000 rpm. | 120 hp (88 kW) at 4000 rpm. |
| EEC peak torque | 350 Nm at 1800-3000 rpm. | 280 Nm at 1500-3000 rpm. |
| Compression ratio | 18:1 | 18:1 |
| Recommended octane rating | Diesel | Diesel |

| Performance | 3.0 V6 TiD | 2.2 TiD |
| --- | --- | --- |
| 0-60 mph (sec) Saloon (Estate | M 8.9 (9.3) A - | M 10.4 (11.3) A 12.5 (13.0) |
| 40–60 mph (sec) in 4th gear Saloon (Estate) | M 6.0 (6.1) | 6.7 (7.1) |
| 50-70 mph (sec) in 5th gear Saloon (Estate) | M 8.8 (9.0) | M 10.0 (11.0) |
| Top speed mph Saloon (Estate) | M 133 (131) A - | M 124 (122) A 122 (118) |

| Fuel consumption Mpg | 3.0 V6 TiD |  | 2.2 TiD |  |
| --- | --- | --- | --- | --- |
|  | M | A | M | A |
| City Saloon | 28.2 | - | 31.4 | 28.5 |
| Estate | 27.4 | - | 30.4 | 27.7 |
| Highway Saloon | 48.7 | - | 54.3 | 47.9 |
| Estate | 47.1 | - | 52.3 | 47.1 |
| Combined Saloon | 38.7 | - | 42.8 | 38.2 |
| Estate | 37.2 | - | 40.1 | 37.2 |
| CO2 Emissions (g/km) | 3.0 V6 TiD |  | 2.2 TiD |  |
|  | M | A | M | A |
| Saloon | 191 | - | 175 | 197 |
| Estate | 202 | - | 178 | 202 |
| Insurance Grouping | 3.0 V6 TiD |  | 2.2 TiD |  |
|  | 15E | 15E | 13E | 13E |